# 荧光定量PCR实验报告

## 实验原理

实时荧光定量PCR（Quantitative Real-time PCR）是一种在DNA扩增反应中，以荧光化学物质测每次聚合酶链式反应（PCR）循环后产物总量的方法。通过内参或者外参法对待测样品中的特定DNA序列进行定量分析的方法。

## 二、实验器材及试剂

### 实验器材

| 名称 | 厂家 | 型号 |
| --- | --- | --- |
| 匀浆仪 | Servicebio | KZ-II |
| 台式高速冷冻离心机 | 海尔 | LX-165T2R |
| 荧光定量PCR仪 | ABI | 7500 |
| 超净工作台 | BIOBASE | BBS-DDC |
| 超微量分光光度计 | 可帮基因 | CANHELP-100 |
| 标准试剂型纯水仪 | 芷昂 | Clever-S |

### 实验试剂

| 试剂 | 厂家 | 货号 |
| --- | --- | --- |
| 总RNA提取剂 | 杭州浩克生物技术有限公司 | HKR022 |
| 三氯甲烷 | 国药集团化学试剂有限公司 | 10006818 |
| 异丙醇 | 国药集团化学试剂有限公司 | 80109218 |
| 无水乙醇 | 国药集团化学试剂有限公司 | 10009218 |
| All-in-One First-Strand cDNA Synthesis SuperMix for qPCR | TRAN | AE341 |
| 2X SYBR Green qPCR Master Mix | APExBIO | K1070 |

## 三、实验方法

### 1. 总RNA抽提（枪头和离心管均经过湿热灭菌，无RNA酶）

- 取匀浆管，加入1mL的Trizol Reagent，置冰上预冷。

- 取100mg组织，加入到匀浆管中。

- 匀浆仪充分研磨直至无可见组织块。

- 12000rpm离心10min取上清。

- 加入250 μL三氯甲烷，颠倒离心管15s，充分混匀，静置3min。

- 4℃ 下13000rpm离心15min。

- 将上清转移到纯化管中，加入1.2倍体积的无水乙醇，混匀。

- 4℃下12000rpm离心5min，纯化管滤膜上的白色沉淀即为RNA。

- 弃去纯化管下部液体，加入75%乙醇650μL 4℃下12000rpm离心5min洗涤。

- 重复洗涤2-3次。

- 4℃下12000rpm离心5min，弃去残留液体，将离心管置于超净台上吹3-5min。

- 加入50μL无RNA酶的水溶解RNA。

- 使用超微量分光光度计检测RNA浓度及纯度：仪器空白调零后取2μL 待测RNA溶液于检测基座上，放下样品臂，使用电脑上的软件开始吸光值检测。

- 将浓度过高的RNA进行适当比例的稀释，使其终浓度为100-500 ng/μL.

### 反转录（枪头和PCR均经过湿热灭菌，无RNA酶）

- 取一PCR管，加入含1μg RNA的溶液。

- 加入1μL dsDNASE 和1μL 10*dsDNase Buffer 。

- 用无核糖核酸酶的去离子水补足至10μL，混匀。

- 于PCR仪上37℃温育2min，以去除基因组DNA污染。65℃温育2min，使dsDNAase失活。

- 加入4μL All-in-One First-Strand Synthesis MasterMix。

- 用无核糖核酸酶的去离子水补足至20μL, 混匀。

- 于PCR仪上50℃孵育15min，结束后85℃孵育5min灭活反转录酶。

### 定量PCR

1）取0.2mL PCR管，配制如下反应体系，每个反转录产物配制3管。

| Reagent | Volume |
| --- | --- |
| cDNA | 1 μL |
| Forward Primer (10 μM) | 0.4 μL |
| Reverse Primer (10 μM) | 0.4 μL |
| Taq SYBR Green qPCR Premix | 10 μL |
| ddH2O | To 20 μL |

- PCR扩增

| - Stage1 预变性 | - Stage2（50个循环） | - Stage3（熔解曲线） |
| --- | --- | --- |
| - 94℃，2min | - 94℃，5s→58℃，10s→72℃，15s | - 60℃→95℃，每10s升温0.5℃ |

### 结果处理

ΔΔCT法：

A=CT(目的基因，待测样本)- CT(内标基因，待测样本)

B=CT(目的基因，对照样本)- CT(内标基因，对照样本)

K=A-B

表达倍数=2-K

# Real Time PCR Report

## Experimental principle

Quantitative Real-time PCR is a method in which a fluorescent chemical is used to measure the total amount of product after each PCR cycle in DNA amplification. A method of quantitative analysis of a specific DNA sequence in a test sample by internal or external parameters.

## Laboratory equipment and reagents

### Laboratory equipment

| Equipment | Manufacturers | Model |
| --- | --- | --- |
| Tissue Grinder | Servicebio | KZ-II |
| High speed refrigerated centrifuge | Haier | LX-165T2R |
| Real time PCR System | ABI | 7500 |
| Clean bench | BIOBASE | BBS-DDC |
| Ultramicro spectrophotometer | Canhelp Genomics Co., Ltd | CANHELP-100 |
| Laboratory pure water system | Zhiang Instrument (Shanghai) Co., Ltd | Clever-S |

### Laboratory reagents

| Reagents | Manufacturers | Catlog |
| --- | --- | --- |
| RNA Extraction | Hangzhou Haoke Biotechnology Co., Ltd | HKR022 |
| trichloromethane | Sinopharm Chemical Reagent co., Ltd. | 10006818 |
| Isopropyl alcohol | Sinopharm Chemical Reagent co., Ltd. | 80109218 |
| Anhydrous ethanol | Sinopharm Chemical Reagent co., Ltd. | 10009218 |
| All-in-One First-Strand cDNA Synthesis SuperMix for qPCR | TRAN | AE341 |
| 2X SYBR Green qPCR Master Mix | APExBIO | K1070 |

## Experimental method

### Isolate RNA

1) Take homogenate tube, add 1mL of Trizol Reagent, place on ice for pre-cooling.

2) Take 100mg tissue and add it to the homogenate tube.

3) The homogenizer fully grinds until no tissue blocks are visible.

4) Centrifuge at 12000rpm for 10min to take the supernatant.

5) Add 250μL trichloromethane, reverse the centrifuge tube for 15s, mix thoroughly, and let stand for 3min.

6) Centrifuge at 13000rpm at 4℃ for 15min.

7) Transfer the supernatant to the purification tube, add 1.2 times the volume of anhydrous ethanol, and mix well.

8) Centrifuge at 12000rpm at 4℃ for 5min, and purify the white precipitate on the tube filter membrane as RNA.

9) Discard the liquid in the lower part of the purification tube, add 75% ethanol 650μL at 4℃, centrifuge at 12000rpm for 5min and wash.

10) Repeat washing 2-3 times.

11) Centrifuge at 12000rpm at 4℃ for 5min, discard the residual liquid, place the centrifuge tube on a super-clean table and blow for 3-5min.

12) Add 50μL of RNA-free water to dissolve RNA.

13) Use an ultramicrospectrophotometer to detect RNA concentration and purity: After blank zero adjustment of the instrument, take 2μL RNA solution to be tested and put it on the detection base, lower the sample arm, and use the software on the computer to start the absorption value detection.

14) The RNA with excessive concentration is diluted in an appropriate proportion so that its final concentration is 100-500 ng/μL.

### First Strand cDNA Synthesis

1) Take a PCR tube and add a solution containing 1μg RNA.

2) Add 1μL dsDNASE and 1μL 10*dsDNase Buffer.

3) Fill with ribonuclease free deionized water to 10μL and mix well.

4) Incubate at 37℃ for 2min on PCR apparatus to remove genomic DNA contamination. dsDNAase was inactivated after incubation at 65℃ for 2min.

5) Add 4μL All-in-One First-Strand Synthesis MasterMix.

6) Fill with ribonuclease free deionized water to 20μL and mix well.

7) Incubation at 50℃ for 15min on the PCR apparatus, and then incubation at 85℃ for 5min to inactivate reverse transcriptase.

### Preparation of PCR Master Mix

1）Take 0.2mL PCR tubes and prepare the reaction system as follows, with 3 tubes for each retroproduct.

| Reagent | Volume |
| --- | --- |
| cDNA | 1 μL |
| Forward Primer (10 μM) | 0.4 μL |
| Reverse Primer (10 μM) | 0.4 μL |
| Taq SYBR Green qPCR Premix | 10 μL |
| ddH2O | To 20 μL |

- PCR amplification

| - Stage1 predegeneration | - Stage2（50cycles） | - Stage3（Melt Curve） |
| --- | --- | --- |
| - 94℃，2min | - 94℃，5s→58℃，10s→72℃，15s | - 60℃→95℃ |

### Data analysis

ΔΔCT method:

A=CT(target gene, sample)- CT(internal standard gene, sample)

B=CT(target gene, control)- CT(internal standard gene, control)

K=A-B

RNA Expression =2-K